NATIONAL OCCUPATIONAL STANDARD

13UY0132-3

WEB OFFSET PRINTING OPERATOR

LEVEL 3

REVISION NO: 00

VOCATIONAL QUALIFICATION AUTHORITY

Ankara, 2013

PREFACE

This reference guide, namely National Qualification of Web Offset Printing Operator (Level 3) was prepared in accordance with the provisions of  the “Regulation on Vocational Qualifications, Testing and Certification” issued pursuant to the Vocational Qualifications Authority (VQA) Law no 5544.

The draft qualification was drawn up by Istanbul Chamber of Commerce appointed upon the cooperation protocol signed on 21.05.2012. After assessing opinions of the relevant organizations and institutions, the draft has been amended accordingly. The final draft was evaluated by the VQA’s Media, Communication, and Publication Sector Committee, which deemed it suitable. It was approved by the Board of Directors of the VQA through its decision no 2013/13 of 06.02.2013 and decided to be placed within the National Qualification Framework (NQF).

We would like to extend our gratitude to all people, organizations and institutions that have expressed their opinions and contributed to the preparation, examination and verification processes of the qualification. We would like to offer it to the service of all likely beneficiaries.

Vocational Qualification Authority

INTRODUCTION

The key criteria referred to in the national qualification preparation process, the relevant sector committees’ review and the VQA’s Board of Directors’ approval processes are set in the Regulation on Vocational Qualification, Testing and Certification.

National qualification is defined by;

a) Name and level of the qualification,

b) Aim and rationale of the qualification,

c) Occupational standard, occupational standard units or Qualification Items which for the basis for the qualification,

ç) Training and experience criteria (form, content, duration etc) necessary for the qualification,

d) Learning outcomes necessary to acquire the qualification,

e) Assessment procedures and principles to be applicable to in the acquisition of the qualification, minimum testing materials and assessor criteria necessary for assessment,

f) Validity of the qualification certificate, renewal conditions, supervision of the certificate holder if deemed necessary,

g) Institution developing the qualification and Sector Committee verifying the Qualification.

National qualifications are built according to the relevant national occupational standard if there is one or to the relevant international occupational standard if there is none at the national level.

National qualifications are set in cooperation with below mentioned organizations;

- Formal and non-formal education and training institutions,

- Authorized Certification Bodies,

- Institutions having preapplied for certification to the authority,

- Institutions having drawn up national occupational standard,

- Professional organizations.

13UY0132-3 WEB OFFSET PRINTING OPERATOR

NATIONAL QUALIFICATION

| 1 | NAME OF QUALIFICATION | Web Offset Printing Operator |
| --- | --- | --- |
| 2 | REFERENCE CODE | 13UY0132-3 |
| 3 | LEVEL | 3 |
| 4 | INTERNATIONAL CLASSIFICATION CODE | ISCO 08: 7322 |
| 5 | TYPE | - |
| 6 | CREDIT VALUE | - |
| 7 | A) DATE OF PUBLICATION | 06.02.2013 |
|  | B) REVISION NO | 00 |
|  | C) REVISION DATE | - |
| 8 | AIM | Aim of this national qualification is to determine if employees and candidates are qualified and having the qualities required for being successful in the profession of Web Offset Printing Operator (Level 3) and to provide them with the opportunity to prove their adequacy in this profession with valid and reliable certificate. This study is also a reference for education system and examination and certification institutions, as well. |
| 9 |  |  |
|  | OCCUPATIONAL STANDARD(S) FORMING THE BASIS FOR THE QUALIFICATION |  |
| 12UMS0183-3 Web Offset Printing Operator (Level 3) National Vocational Standard 12UMS0184-3 Sheet-fed Offset Printing Operator (Level 3) National Vocational Standard |  |  |
| 10 |  |  |
|  | REQUIREMENTS TO TAKE QUALIFICATION TEST |  |
| - |  |  |
| 11 |  |  |
|  | STRUCTURE OF QUALIFICATION |  |
| 11-a) Compulsory Units |  |  |
| 13UY0133-3/A1: Occupational Health and Safety, Environmental Safety, Quality 13UY0133-3/A2: Work Organization and Occupational Development 13UY0133-3/A3: Offset Printing Materials 13UY0132-3/A4: Knowing Offset Printing System and Web Offset Printing Machinery and Maintenance Applications 13UY0132-3/A5: Web Offset printing Preparations and Printing Applications |  |  |

| 11-b) Elective Units |  |  |  |  |
| --- | --- | --- | --- | --- |
| - |  |  |  |  |
| 11-c) Alternatives for Grouping of Units and Additional Learning Outcomes |  |  |  |  |
| The candidate should be successful in all defined compulsory qualification items in certification of Web Offset Printing Operator (Level 3) occupation. |  |  |  |  |
| 12 |  |  |  |  |
|  | ASSESSMENT |  |  |  |
| Theoretical Assessment: Theoretical assessment of qualification items is made by means of written exam, which may be conducted separately or collectively. Being successful in theoretical exam is prerequisite for taking performance based examination. Assessment tools, which include certain number of multiple-choice and equal-score questions to be answered within predefined time, shall be used in theoretical assessment of Web Offset Printing Operator (Level 3) vocational Qualification Items. Performance Based Assessment: Performance based examination for assessing performance of candidates who apply for certification in occupation of Web Offset Printing Operator (Level 3) shall be made in real or realistically arranged working environment and by using the printing machine, apparatus and hardware, raw materials, and auxiliary materials. Performance based questions shall be prepared in the manner of applications to measure process and results, scenarios fictionalized in accordance with scale, and formats containing critical conditions. Scores, outcomes, and / or process stages corresponding to learning outcomes and performance criteria required by qualification item shall be recorded in the checklists, whose (when required) times and critical actions were defined and arranged in accordance with performances specified in relevant items in conjunction with observations, remarks, assessment, and scoring records. During performance based examination the candidates shall be ensured to use personal protective equipment (PPE) in order to ensure protection against risks, which are not possible to prevent. Candidates who do not comply with safety rules of occupation during performance based examination cannot continue the examination. General Conditions: In theoretical and performance based examinations qualifications of candidates shall be assessed by using question banks developed in accordance with learning outcomes and performance criteria defined as per items. Degree of difficulty of questions to ask during assessment shall be determined in accordance with the level of qualification. Candidates who cannot be successful in all items shall have the right to take examination within 1 (one) year for the items which they fail and be granted with the certificate of achievement for the items they pass. |  |  |  |  |
| 13 | VALIDITY OF CERTIFICATE |  |  | Valid for five (5) years beginning from date of issue. |
| 14 | FREQUENCY OF SURVEILLANCE |  |  | Candidate shall be subject to surveillance at least once within validity period of the certificate by authorized certification institute. |
| 15 | ASSESSMENT METHODS TO BE FOLLOWED IN RENEWAL OF EXPIRED CERTIFICATES |  |  | At the end of validity period surveillance shall be conducted for renewal purpose. If candidate can document that s/he has worked as Web Offset Printing Operator (Level 3) for at least 18 months and s/he has not suspended the occupation more than 18 months and if no change or revision has been made in qualification the certificate of applicant whom performance was assessed positively in aforementioned surveillance shall be extended by 5 (five) more years without needing any additional examination. Performance based examination shall be repeated for those who cannot be successful or submit evidence for their work in surveillance. |
|  |  |  | In case of any change in qualification the candidates shall be subject to theoretical and practical exam for changed unit/units. |  |
| 16 |  | QUALIFICATION DEVELOPMENT INSTITUTION(S) | Istanbul Chamber of Commerce (ITO) |  |
| 17 |  | SECTOR COMMITTEE TO VERIFY QUALIFICATION | VQA Media, Communication, and Publication Sector Committee |  |
| 18 |  | DATE AND NUMBER OF APPROVAL BY VQA BOARD OF DIRECTORS | 06.02.2013-2013/13 |  |

13UY0133-3/A1 OCCUPATIONAL HEALTH AND SAFETY, ENVIRONMENTAL SAFETY, QUALITY

QUALIFICATION ITEM

| 1 | NAME OF QUALIFICATION ITEM |  | Occupational Health and Safety, Environmental Safety, Quality |  |
| --- | --- | --- | --- | --- |
| 2 | REFERENCE CODE |  | 13UY0133-3/A1 |  |
| 3 | LEVEL |  | 3 |  |
| 4 | CREDIT VALUE |  | - |  |
| 5 | A) DATE OF PUBLICATION |  | 06.02.2013 |  |
|  | B) REVISION NO |  | 00 |  |
|  | C) REVISION DATE |  | - |  |
| 6 | OCCUPATIONAL STANDARD(S) FORMING THE BASIS FOR THE QUALIFICATION |  |  |  |
| 12UMS0184-3 Sheet-fed Offset Printing Operator (Level 3) National Vocational Standard |  |  |  |  |
| 7 | LEARNING OUTCOMES |  |  |  |
| Learning Outcome 1: Applies OHS instructions in workplace. Performance Criteria: 1.1. Defines basic risks related to OHS, environmental safety, and quality. 1.2. Defines instructions related to the OHS. - Removes any and all accessories which s/he wears on her/his neck or hand in the workplace. - Uses appropriate Personal Protective Equipment (PPE) in workplace and printing processes. - Applies the measures defined in instructions in accordance with risks and properties of chemicals used in workplace and printing processes. Context: 1.1-1.2: OHS legislation regarding with printing works, technical documents and user manuals of offset printing machines, OHS, environmental safety, and quality instructions of enterprises 1.3: Accessories to take off prior to printing: watch, ring, bracelet, wristband, tie, foulard, necklace, etc. Any and all kinds of accessories to wear on arm, finger, neck, etc. 1.4: Essential PPE's used in printing processes (earplug, steel toe shoes, mask, working clothes, gloves) 1.5: Material safety information forms as well as instructions regarding chemicals used in printing process. Learning Outcome 2: Obeys environmental safety instructions in the workplace. Performance Criteria: 2.1. Defines environmental safety instructions. 2.2. Defines wastes and waste materials which are risky from view point of environmental safety in printing processes. 2.3. Applies the measures regarding classification and disposal of wastes originating from printing operations. Context: 2.1: Environmental legislation, instructions regarding classification and disposal of wastes originating from printing operations 2.2-2.3: Applications relating to classification of wastes originating from printing process as recyclable or non-recyclable or according to material properties and being labeled, stored and transported of wastes in appropriate environments. Learning Outcome 3: Applies emergency instructions in the workplace. Performance Criteria: 3.1. Carries out necessary notifications appropriately in accordance with instructions in case of work accidents. 3.2. Carries out the tasks assigned to printing operators within frame of printing environment emergency procedures. |  |  |  |  |
| 3.3. Carries out basic activities regarding first response to fire in case of any fire in defined conditions (an object in fire, source of fire, etc) in printing environment. Context: 3: Work accidents and emergency instructions within frame of OHS legislation, emergency procedures regarding response to fire, and first aid rules Learning Outcome 4: Contributes to improvement of work processes and productivity. Performance Criteria: Defines efficient working elements in workplace and work processes. Defines quality instructions according to efficient working elements in workplace and work processes. Context: 4: Quality and productivity instructions and criteria in printing processes (wastage amounts and ratios, material saving, energy efficiency, stoppages, etc.), printing process workflow diagrams and work definitions, standards regarding offset printing. |  |  |  |  |
| 8 |  | ASSESSMENT |  |  |
| 8 a) Theoretical Examination |  |  |  |  |
| Theoretical assessment in Occupational Health and Safety and Environmental Safety qualification item shall be conducted by means of written examination consisting of at least 8 multiple-choice and equal-score questions (T1). Values and difficulty levels of questions in assessment scale shall be determined in accordance with cognitive level. The candidate shall be given at least 1.5 minutes to answer each question. Questions shall be determined so as to cover all performance criteria. No penalty shall be applied for wrong answers. The candidate should score at least 60 /100 in order to be successful in theoretical assessment of this unit. |  |  |  |  |
| 8 b) Performance based Examination |  |  |  |  |
| Performance of candidates in Occupational Health and Safety and Environmental Safety qualification unit shall be assessed by means of “application checklist” developed for assessing the process and results (P1) after observing their attitudes and behaviors defined in respect with learning outcomes and performance criteria in real or realistically arranged working environment. The candidate should achieve adequate performance in all of critical steps defined in checklist and score at least 80/100 in order to be successful in performance based examination. |  |  |  |  |
| 8 c) Other Assessment related conditions |  |  |  |  |
| - |  |  |  |  |
| 9 |  | QUALIFICATION DEVELOPMENT INSTITUTION(S)/ORGANIZATIONS(S) |  | Istanbul Chamber of Trade (ITO) |
| 10 |  | SECTOR COMMITTEE TO VERIFY QUALIFICATION |  | VQA Media, Communication, and Publication Sector Committee |
| 11 |  | DATE AND NUMBER OF APPROVAL BY VQA BOARD OF DIRECTORS |  | 06.02.2013-2013/13 |

ANNEXES

ANNEX 1: Information on Recommended Training to Earn a Qualification Unit

1. Occupational Health and Safety Legislation

2. OHS and environmental safety risks related to offset printing process, materials, machinery, and equipment

3. Personal safety and occupational health and safety in offset printing process

4. Safety and safe use of materials, machinery, and equipment in offset printing process

5. Safety- and production-related environmental arrangements in offset printing process

6. Environmental legislation and environmental protection

7. Environmental protection and waste disposal applications in offset printing processes

8. Emergency applications in offset printing environment

9. First aid

10. Offset printing process and productivity

11. National and international quality standards of offset printing

12. Basic quality management

13. Hygiene rules

ANNEX 2(*): Table Regarding Performance Criteria Measured by Means of Assessment Tools Specified in Qualification Unit

| Performance Criteria |  |  | Assessment Tools |
| --- | --- | --- | --- |
| 1 | 1.1 | Defines basic risks related to OHS, environmental safety, and quality. | T1 |
| 2 | 1.2 | Defines instructions related to the OHS. | T1 |
| 3 | 1.3 | Removes any and all accessories which s/he wears on her/his neck or hand in the workplace. | P1 |
| 4 | 1.4 | Uses appropriate Personal Protective Equipment (PPE) in workplace and printing processes. | T1, P1 |
| 5 | 1.5 | Applies the measures defined in instructions in accordance with risks and properties of chemicals used in workplace and printing processes. | T1, P1 |
| 6 | 2.1 | Defines environmental safety instructions. | T1 |
| 7 | 2.2 | Defines wastes and waste materials which are risky from view point of environmental safety in printing processes. | T1 |
| 8 | 2.3 | Applies the measures regarding classification and disposal of wastes originating from printing operations. | T1, P1 |
| 9 | 3.1 | Carries out necessary notifications appropriately in accordance with instructions in case of work accidents. | T1 |
| 10 | 3.2 | Carries out the tasks assigned to printing operators within frame of printing environment emergency procedures. | T1 |
| 11 | 3.3 | Carries out basic activities regarding first response to fire in case of any fire in defined conditions (an object in fire, source of fire, etc) in printing environment. | T1 |
| 12 | 4.1 | Defines efficient working elements in workplace and work processes. | T1 |
| 11 | 4.2 | Defines quality instructions according to efficient working elements in workplace and work processes. | T1 |

(*): These annexes will be reserved for assessment of draft qualifications and/or authorized institutions and will not be made public.

13UY0133-3/A2 WORK ORGANIZATION AND OCCUPATIONAL DEVELOPMENT QUALIFICATION ITEM

| 1 | NAME OF QUALIFICATION ITEM |  | Work Organization and Occupational Development |  |
| --- | --- | --- | --- | --- |
| 2 | REFERENCE CODE |  | 13UY0133-3/A2 |  |
| 3 | LEVEL |  | 3 |  |
| 4 | CREDIT VALUE |  | - |  |
| 5 | A) DATE OF PUBLICATION |  | 06.02.2013 |  |
|  | B) REVISION NO |  | 00 |  |
|  | C) REVISION DATE |  | - |  |
| 6 | OCCUPATIONAL STANDARD(S) FORMING THE BASIS FOR THE QUALIFICATION |  |  |  |
| 12UMS0184-3 Sheet-fed Offset Printing Operator (Level 3) National Vocational Standard |  |  |  |  |
| 7 | LEARNING OUTCOMES |  |  |  |
| Learning Outcome 1: Carries out work commencing and completing activities. Performance Criteria: 1.1. Defines work steps regarding shift changes. 1.2. Defines work steps regarding work order receiving, work finalizing, and giving and receiving purchase order information. Context: 1: Work definitions of printing operators and printing assistants, printing processes workflow diagrams, printing machine operating instructions. Learning Outcome 2: Keeps records of her/his tasks during work process. Performance Criteria: 2.1. Keeps printing machine maintenance forms in accordance with instructions thereof. 2.2. Defines relevant records and samples put on print stacks. Context: 2: Work order / work definition files during printing process, printing machine operation, control, maintenance, and repair forms, types of documents put onto printed product stacks (mock-up, blueprint, page proof, record form, etc.), and instructions regarding records of printing processes and machines. Learning Outcome 3: Carries out her/his occupational development. Performance Criteria: 3.1. Defines occupational development monitoring resources. Context: 3: Printing fairs and exhibitions, websites and periodicals on offset printing, |  |  |  |  |
| 8 | ASSESSMENT |  |  |  |
| 8 a) Theoretical Examination |  |  |  |  |
| Theoretical assessment in Work Organization and Occupational Development Qualification Item shall be conducted by means of written examination consisting of at least 4 multiple-choice and equal-score questions (T1). Values and difficulty levels of questions in assessment scale shall be determined in accordance with cognitive level. The candidate shall be given at least 1.5 minutes to answer each question. Questions shall be determined so as to cover all performance criteria. No penalty shall be applied for wrong answers. |  |  |  |  |
| The candidate should score at least 60 /100 in order to be successful in theoretical assessment of this unit. |  |  |  |  |
| 8 b) Performance based Examination |  |  |  |  |
| There is no performance assessment for Work Organization and Occupational Development Qualification Item. |  |  |  |  |
| 8 c) Other Assessment related conditions |  |  |  |  |
| - |  |  |  |  |
| 9 |  | QUALIFICATION DEVELOPMENT INSTITUTION(S)/ORGANIZATIONS(S) |  | Istanbul Chamber of Commerce (ITO) |
| 10 |  | SECTOR COMMITTEE TO VERIFY QUALIFICATION |  | VQA Media, Communication, and Publication Sector Committee |
| 11 |  | DATE AND NUMBER OF APPROVAL BY VQA BOARD OF DIRECTORS |  | 06.02.2013-2013/13 |

ANNEXES

ANNEX1  Information on Recommended Training to Earn a Qualification Unit

1. Basic labor legislation

2. Offset printing work processes and process monitoring (technically)

3. Keeping record during offset printing work processes

4. Resources of vocational information on area of printing and offset printing

5. Communication skills

6. Ability of working within a team

ANNEX 2(*) 	Table Regarding Performance Criteria Measured by Means of Assessment Tools Specified in Qualification Unit

| Performance Criteria |  |  | Assessment Tools |
| --- | --- | --- | --- |
| 1 | 1.1 | Defines work steps regarding shift changes. | T1 |
| 2 | 1.2 | Defines work steps regarding work order receiving, work finalizing, and giving and receiving purchase order information. | T1 |
| 3 | 2.1 | Keeps printing machine maintenance forms in accordance with instructions thereof. | T1 |
| 4 | 2.2 | Defines relevant records and samples put on print stacks. | T1 |
| 5 | 3.1 | Defines occupational development monitoring resources. | T1 |

(*): These annexes will be reserved for assessment of draft qualifications and/or authorized institutions and will not be made public.

13UY0133-3/A3: OFFSET PRINTING MATERIALS QUALIFICATION ITEM

| 1 | NAME OF QUALIFICATION ITEM | Offset Printing Materials |  |
| --- | --- | --- | --- |
| 2 | REFERENCE CODE | 13UY0133-3/A3 |  |
| 3 | LEVEL | 3 |  |
| 4 | CREDIT VALUE | - |  |
| 5 | A) DATE OF PUBLICATION | 06.02.2013 |  |
|  | B) REVISION NO | 00 |  |
|  | C) REVISION DATE | - |  |
| 6 | OCCUPATIONAL STANDARD(S) FORMING THE BASIS FOR THE QUALIFICATION |  |  |
| 12UMS0184-3 Sheet-fed Offset Printing Operator (Level 3) National Vocational Standard |  |  |  |
| 7 | LEARNING OUTCOMES |  |  |
| Learning Outcome 1: Distinguishes properties and types of paper. Performance Criteria: 1.1. Distinguishes paper types from each other. 1.2. Measures weight in grams and thickness of paper to be used in printing by means of appropriate tools. 1.3. Determines waterway of paper to be used in printing. 1.4. Defines standard paper sizes. 1.5. Defines appropriate conditions to store the paper. Context: 1: Paper standards and types (high-grade, lightly-glazed, glossy paper, etc.), paper measuring tools, instructions regarding paper and storage conditions of paper Learning Outcome 2: Knows properties of press moulds. Performance Criteria: 2.1. Determines mould sizes in accordance with printing machine. 2.2. Determines suitability of moulds for printing in physical and functional manner. 2.3. Determines plinth and shear direction of moulds to be used in printing. Context: 2: Physical and functional properties of the mould (color information and values, dot value, scratch, smash, etc. information) Learning Outcome 3: Determines appropriateness of blanket to the machine. Performance Criteria: 3.1. Detects suitability of blanket sizes to machinery. 3.2. Detects suitability of blanket and feeder thickness to the machinery. Context: 3: Printing machines technical documentation Learning Outcome 4: Knows properties of printing inks and colors. Performance Criteria: 4.1. Defines ink types used in offset printing. 4.2. Knows CMYK, pantone, and other colors. Context: 4: Content of product introduction documents regarding with printing ink, ink production standard, offset printing standards, CMYK colors, pantone color formulation system (catalogue) Learning Outcome 5: Knows properties of chemicals and auxiliary materials. Performance Criteria: 5.1. Defines reservoir water. 5.2. Defines types and intended uses of chemical materials used in offset printing. 5.3. Explains auxiliary materials used in offset printing by functions. Context: 5: Reservoir water concentration, IPA (alcohol), fixer, solvents, water hardness regulator, corrector, glue, sprayed powder, rolling press cleaning pastes, blanket cleaners, etc. |  |  |  |
| 8 | ASSESSMENT |  |  |
| 8 a) Theoretical Examination |  |  |  |
| Theoretical assessment in Offset Printing Materials qualification unit shall be conducted by means of written examination consisting of at least 10 multiple-choice and equal-score questions (T1). Values and difficulty levels of questions in assessment scale shall be determined in accordance with cognitive level. The candidate shall be given at least 1.5 minutes to answer each question. Questions shall be determined so as to cover all performance criteria. No penalty shall be applied for wrong answers. The candidate should score at least 60 /100 in order to be successful in theoretical assessment of this unit. |  |  |  |
| 8 b) Performance based Examination |  |  |  |
| Performance of candidates in Offset Printing Materials qualification item shall be assessed by means of “application checklist” developed for assessing the process and results (P1) after observing their attitudes and behaviors defined in respect with learning outcomes and performance criteria in real or realistically arranged working environment. The candidate should achieve adequate performance in all of critical steps defined in checklist and score at least 80/100 in order to be successful in performance based examination. |  |  |  |
| 8 c) Other Assessment related conditions |  |  |  |
| - |  |  |  |
| 9 | QUALIFICATION DEVELOPMENT INSTITUTION(S)/ORGANIZATIONS(S) |  | Istanbul Chamber of Trade (ITO) |
| 10 | SECTOR COMMITTEE TO VERIFY QUALIFICATION |  | VQA Media, Communication, and Publication Sector Committee |
| 11 | DATE AND NUMBER OF APPROVAL BY VQA BOARD OF DIRECTORS |  | 06.02.2013-2013/13 |

ANNEXES

ANNEX 1 Information on Recommended Training to Earn a Qualification Unit

1. Basic properties of printing papers and paper measurement

2. Basic properties of printing moulds (including mould direction) and basic knowledge on mould production (analogue, digital)

3. Basic properties of blanket and placement into printing machine

4. Basic properties of ink used in offset printing

5. Colors in offset printing

6. Basic knowledge on chemistry regarding and chemicals used in offset printing operations

7. Auxiliary materials used in offset printing

ANNEX 2(*) Table Regarding Performance Criteria Measured by Means of Assessment Tools Specified in Qualification Unit

| Performance Criteria |  |  | Assessment Tools |
| --- | --- | --- | --- |
| 1 | 1.1 | Distinguishes paper types from each other. | P1 |
| 2 | 1.2 | Measures weight in grams and thickness of paper to be used in printing by means of appropriate tools. | T1, P1 |
| 3 | 1.3 | Determines waterway of paper to be used in printing. | T1, P1 |
| 4 | 1.4 | Defines standard paper sizes. | T1 |
| 5 | 1.5 | Defines appropriate conditions to store the paper. | T1 |
| 6 | 2.1 | Determines mould sizes in accordance with printing machine. | P1 |
| 7 | 2.2 | Determines suitability of moulds for printing in physical and functional manner. | P1 |
| 8 | 2.3 | Determines plinth and shear direction of moulds to be used in printing. | T1, P1 |
| 9 | 3.1 | Detects suitability of blanket sizes to machinery. | P1 |
| 10 | 3.2 | Detects suitability of blanket and feeder thickness to the machinery. | P1 |
| 11 | 4.1 | Defines ink types used in offset printing. | T1 |
| 12 | 4.2 | Knows CMYK, pantone, and other colors. | T1 |
| 13 | 5.1 | Defines reservoir water. | T1 |
| 14 | 5.2 | Defines types and intended uses of chemical materials used in offset printing. | T1 |
| 15 | 5.3 | Explains auxiliary materials used in offset printing by functions. | T1 |

(*): These annexes will be reserved for assessment of draft qualifications and/or authorized institutions and will not be made public.

13UY0132-3/A4 KNOWING AND PERFORMING MAINTENANCE OF OFFSET PRINTING SYSTEM AND WEB OFFSET PRINTING MACHINE QUALIFICATION ITEM

| 1 | NAME OF QUALIFICATION ITEM | Knowing Offset Printing System and Web Offset Printing Machinery and Maintenance Applications |  |
| --- | --- | --- | --- |
| 2 | REFERENCE CODE | 13UY0132-3/A4 |  |
| 3 | LEVEL | 3 |  |
| 4 | CREDIT VALUE | - |  |
| 5 | A) DATE OF PUBLICATION | 06.02.2013 |  |
|  | B) REVISION NO | 00 |  |
|  | QREVIZYON TARİHİ | - |  |
| 6 | OCCUPATIONAL STANDARD(S) FORMING THE BASIS FOR THE QUALIFICATION |  |  |
| 12UMS0183-3 Web Offset Printing Operator (Level 3) National Vocational Standard |  |  |  |
| 7 | LEARNING OUTCOMES |  |  |
| Learning Outcome 1: Knows basic properties of offset printing system Performance Criteria: 1.1. Explains offset printing principle. 1.2. Explains basic properties of offset printing system Context: 1: Offset printing system properties (indirect, straight, humidifying system, being oil-based of offset inks, etc), offset printing principle (lithography) Learning Outcome 2: Knows basic properties and working principles of web offset printing machines. Performance Criteria: 2.1. Defines printing machine inlet system, printing system, ink system, humidifying system, weighting pressure, drying systems, outlet system, control desk, safety system, and functions thereof. 2.2. Defines auxiliary apparatus used in printing machine. Context: 2: Technical documents on web offset printing system and printing system auxiliary apparatus Learning Outcome 3: Performs cleaning and lubricating operations of web offset printing machine. Performance Criteria: 3.1. Cleans printing machine blanket, ink tank and rolling presses, humidifying system, inlet and outlet system, and photocells by means of appropriate chemical materials in accordance with their technical properties. 3.2. Lubricates required places of printing machine and auxiliary equipment as specified in protective maintenance plan and defined in maintenance manual. 3.3. Checks oil levels of machine compressors and automatic lubricating systems from oil indicators. 3.4. Supplies lubricant for required places of printing machine and auxiliary equipment as specified in protective maintenance plan and defined in maintenance manual. Context: 3: Technical documents, user manuals, and operation and maintenance manuals of web offset printing machines and auxiliary equipment. |  |  |  |
| Learning Outcome 4: Performs routine controls of web offset printing machine. Performance Criteria: 4.1. Performs routine controls of rolling press surfaces, blankets, press-bands, humidifying filters and water, and cooling and drying system in accordance with physical characteristics. 4.2. Performs safety controls of pressing machine in accordance with instructions. 4.3. Explains reasons of pressing machine and auxiliary equipment safety controls. Context: 4: Technical documents and user manuals of web offset printing machines and auxiliary equipment. |  |  |  |
| 8 | ASSESSMENT |  |  |
| 8 a) Theoretical Examination |  |  |  |
| Theoretical assessment in Knowing and Performing Maintenance of Offset Printing System and Web Offset Printing Machine qualification item shall be conducted by means of written examination consisting of at least 10 multiple-choice and equal-score questions (T1). Values and difficulty levels of questions in assessment scale shall be determined in accordance with cognitive level. The candidate shall be given at least 1.5 minutes to answer each question. Questions shall be determined so as to cover all performance criteria. No penalty shall be applied for wrong answers. The candidate should score at least 60 /100 in order to be successful in theoretical assessment of this unit. |  |  |  |
| 8 b) Performance based Examination |  |  |  |
| Performance of candidates in Knowing and Performing Maintenance of Offset Printing System and Web Offset Printing Machine qualification item shall be assessed by means of “application checklist” developed for assessing the process and results (P1) after observing their attitudes and behaviors defined in respect with learning outcomes and performance criteria in real or realistically arranged working environment. The candidate should achieve adequate performance in all of critical steps defined in checklist and score at least 80/100 in order to be successful in performance based examination. |  |  |  |
| 8 c) Other Assessment related conditions |  |  |  |
| - |  |  |  |
| 9 | QUALIFICATION DEVELOPMENT INSTITUTION(S)/ORGANIZATIONS(S) |  | Istanbul Chamber of Commerce (ITO) |
| 10 | SECTOR COMMITTEE TO VERIFY QUALIFICATION |  | VQA Media, Communication, and Publication Sector Committee |
| 11 | DATE AND NUMBER OF APPROVAL BY VQA BOARD OF DIRECTORS |  | 06.02.2013-2013/13 |

ANNEXES

ANNEX  Information on Recommended Training to Earn a Qualification Unit

1. General information on offset printing system (lithography)

2. Basic electricity and electronic knowledge

3. Knowledge of basic mechanics

4. Web offset printing machines technology (technical properties at basic level)

5. Web offset printing machine inlet system, printing system, ink system, humidifying system, weighting pressure, drying systems, outlet system, control desk, safety system, and functions thereof.

6. Auxiliary apparatus and equipment used in web offset printing machines

7. Cleaning and maintenance of web offset printing machine

8. Routine controls of web offset printing machine (including safety controls)

ANNEX 2(*) Table Regarding Performance Criteria Measured by Means of Assessment Tools Specified in Qualification Unit

| Performance Criteria |  |  | Assessment Tools |
| --- | --- | --- | --- |
| 1 | 1.1 | Explains offset printing principle. | T1 |
| 2 | 1.2 | Explains basic properties of offset printing system | T1 |
| 3 | 2.1 | Defines printing machine inlet system, printing system, ink system, humidifying system, weighting pressure, drying systems, outlet system, control desk, safety system, and functions thereof. | T1 |
| 4 | 2.2 | Defines auxiliary apparatus used in printing machine. T1 | T1 |
| 5 | 3.1 | Cleans printing machine blanket, ink tank and rolling presses, humidifying system, inlet and outlet system, and photocells by means of appropriate chemical materials in accordance with their technical properties. | T1, P1 |
| 6 | 3.2 | Lubricates required places of printing machine and auxiliary equipment as specified in protective maintenance plan and defined in maintenance manual. | T1, P1 |
| 7 | 3.3 | Checks oil levels of machine compressors and automatic lubricating systems from oil indicators. | P1 |
| 8 | 3.4 | Supplies lubricant for required places of printing machine and auxiliary equipment as specified in protective maintenance plan and defined in maintenance manual. | T1 |
| 9 | 4.1 | Performs routine controls of rolling press surfaces, blankets, press-bands, humidifying filters and water, and cooling and drying system in accordance with physical characteristics. | T1, P1 |
| 10 | 4.2 | Performs safety controls of pressing machine in accordance with instructions. | P1 |
| 11 | 4.3 | Explains reasons of pressing machine and auxiliary equipment safety controls. | T1 |

(*): These annexes will be reserved for assessment of draft qualifications and/or authorized institutions and will not be made public.

13UY0132-3/A5 WEB OFFSET PRINTING PREPARATIONS AND PRINTING APPLICATIONS

QUALIFICATION ITEM

| 1 | NAME OF QUALIFICATION ITEM | Web Offset Printing Preparations and Printing Applications |
| --- | --- | --- |
| 2 | REFERENCE CODE | 13UY0132-3/A5 |
| 3 | LEVEL | 3 |
| 4 | CREDIT VALUE | - |
| 5 | A) DATE OF PUBLICATION | 06.02.2013 |
|  | B) REVISION NO | 00 |
|  | C) REVISION DATE | - |
| 6 | OCCUPATIONAL STANDARD(S) FORMING THE BASIS FOR THE QUALIFICATION |  |
| 12UMS0183-3 Web Offset Printing Operator (Level 3) National Vocational Standard |  |  |
| 7 | LEARNING OUTCOMES |  |
| Learning Outcome 1: Activates printing machines. Performance Criteria: 1.1. Activates printing machine by energizing. 1.2. Ensures operability of machinery and hardware by monitoring indicators and warning lamps of the machine. 1.3. Ensures cleanness of printing machine by performing before-printing cleaning controls. 1.4. Arranges machine size and machine apparatus in accordance with work order when passing from one work to another. Context: 1: Technical documents and user manuals of web offset printing machines and auxiliary equipment. Learning Outcome 2: Activates humidifying unit of printing machine. Performance Criteria: 2.1. Opens humidifying unit of printing machine. 2.2. Cleans dirty filters. 2.3. Replaces deformed filters. Context: 2: Technical documents of machine and humidifying unit. Learning Outcome 3: Loads ink into printing machine in accordance with defined order of colors. Performance Criteria: 3.1. Defines color order in offset printing. 3.2. Places the inks into tanks of machine in defined order. Context: 3: Offset printing standards (ISO 12647-2 and ISO 12647-3), documents about the ink, ink production standard. Learning Outcome 4: Prepares paper systems of printing machine for printing process. Performance Criteria: 4.1. Checks suitability of paper roll for defined work. 4.2. Inserts paper roll to roll stand of machine in accordance with amount of work to print and number of pages. |  |  |

| 4.3. Lays the paper into printing units. 4.4. Applies folding pattern on the machine in accordance with the work to print. Context: 4: Technical documents of web offset printing machine, folding patterns according to work, work order Learning Outcome 5: Puts the mould into printing machine. Performance Criteria: 5.1. Prepares punches and folds of mould to be used in printing in accordance with plinth and shear direction. 5.2. Resets printing machine registers. 5.3. Inserts the moulds into printing machine units in order of color. Context: 5: Technical documents and manuals of printing machine Learning Outcome 6: Follows the printing process of printing machine. Performance Criteria: 6.1. Adjusts numerator of machine according to amount of print specified in the work order. 6.2. Supervises proper entering and flow of paper into printing process by controlling folding crosses. 6.3. Carries out physical controls of the resulting work at control desk. 6.4. Assesses the problems such as shading, scratching, change in color values, burring, adhesion, drying problems, posing and shield disproportion, contaminating, static electricity in paper, etc, originating at printing processes. 6.5. Stops the machine in accordance with instructions when planned printing is completed. 6.6. Pulls printed stacks into semi-product area and puts work / purchase order files and printing samples onto stacks. Context: 6: Illuminated control desk, offset printing standards (ISO 12647-2 and ISO 12647-3), printing control strips (doubling-slipping, gray balance, trapping, etc.), guides (crosses), work orders, spectrophotometer, density meter, front-back print, operating procedures. |  |  |  |
| --- | --- | --- | --- |
| 8 | ASSESSMENT |  |  |
| 8 a) Theoretical Examination |  |  |  |
| Theoretical assessment in Web Offset Printing Preparations and Printing Applications qualification unit shall be conducted by means of written examination consisting of at least 10 multiple-choice and equal-score questions (T1). Values and difficulty levels of questions in assessment scale shall be determined in accordance with cognitive level. The candidate shall be given at least 1.5 minutes to answer each question. Questions shall be determined so as to cover all performance criteria. No penalty shall be applied for wrong answers. The candidate should score at least 60 /100 in order to be successful in theoretical assessment of this unit. |  |  |  |
| 8 b) Performance based Examination |  |  |  |
| Performance of candidates in Web Offset Printing Preparations and Printing Applications qualification unit shall be assessed by means of “application checklist” developed for assessing the process and results (P1) after observing their attitudes and behaviors defined in respect with learning outcomes and performance criteria in real or realistically arranged working environment. The candidate should achieve adequate performance in all of critical steps defined in checklist and score at least 80/100 in order to be successful in performance based examination. |  |  |  |
| 8 c) Other Assessment related conditions |  |  |  |
| - |  |  |  |
| 9 |  | QUALIFICATION DEVELOPMENT INSTITUTION(S)/ORGANIZATIONS(S) | Istanbul Chamber of Commerce (ITO) |
| 10 |  | SECTOR COMMITTEE TO VERIFY QUALIFICATION | VQA Media, Communication, and Publication Sector Committee |
| 11 |  | DATE AND NUMBER OF APPROVAL BY VQA BOARD OF DIRECTORS | 06.02.2013-2013/13 |

ANNEXES

ANNEX1 Information on Recommended Training to Earn a Qualification Unit

1. Knowledge of occupational terminology related to web offset printing processes

2. Basic knowledge on offset printing processes and graphical processes

3. Applications of preparing web offset printing machine for printing process

4. Ability of using printing control equipment

5. Knowledge and ability of carrying out physical controls of printing outputs

6. Various printing applications in web offset printing machine

ANNEX 2 (*) Table Regarding Performance Criteria Measured by Means of Assessment Tools Specified in Qualification Unit

| Performance Criteria |  |  | Assessment Tools |
| --- | --- | --- | --- |
| 1 | 1.1 | Defines phases of activating printing machine by energizing. | T1 |
| 2 | 1.2 | Ensures operability of machinery and hardware by monitoring indicators and warning lamps of the machine. | P |
| 3 | 1.3 | Ensures cleanness of printing machine by performing before-printing cleaning controls. | P1 |
| 4 | 1.4 | Arranges machine size and machine apparatus in accordance with work order when passing from one work to another. | T1 |
| 5 | 2.1 | Opens humidifying unit of printing machine. | P1 |
| 6 | 2.2 | Cleans dirty filters. | P1 |
| 7 | 2.3 | Determines replacing steps of deformed filters. | T1 |
| 8 | 3.1 | Defines color order in offset printing. | T1 |
| 9 | 3.2 | Defines methods of being placed of inks into tanks of machine in defined order. | T1 |
| 10 | 4.1 | Explains how to determine suitability of paper roll for defined work. | T1 |
| 11 | 4.2 | Inserts paper roll to roll stand of machine in accordance with amount of work to print and number of pages. | P1 |
| 12 | 4.3 | Lays the paper into printing units. | P1 |
| 13 | 4.4 | Explains how to apply folding pattern on the machine in accordance with the work to print. | T1 |
| 14 | 5.1 | Prepares punches and folds of mould to be used in printing in accordance with plinth and shear direction. | P1 |
| 15 | 5.2 | Resets printing machine registers. | P1 |
| 16 | 5.3 | Inserts the moulds into printing machine units in order of color. | P1 |
| 17 | 6.1 | Explains how to control numerator of machine according to amount specified in work order. | T1 |
| 18 | 6.2 | Supervises proper entering and flow of paper into printing process by controlling folding crosses. | P1 |
| 19 | 6.3 | Defines what types of physical controls shall be made on resulting work on control desk. | T1 |
| 20 | 6.4 | Assesses the problems such as shading, scratching, change in color values, burring, adhesion, drying problems, posing and shield disproportion, contaminating, static electricity in paper, etc, originating at printing processes. | T1, P1 |
| 21 | 6.5 | Stops the machine in accordance with instructions when planned printing is completed. | T1 |
| 22 | 6.6 | Pulls printed stacks into semi-product area and puts work / purchase order files and printing samples onto stacks. | P1 |

(*): These annexes will be reserved for assessment of draft qualifications and/or authorized institutions and will not be made public.

QUALIFICATION ANNEXES

ANNEX 1: Qualification Items

13UY0133-3/A1: Occupational Health and Safety, Environmental Safety, Quality

13UY0133-3/A2: Work Organization and Occupational Development

13UY0133-3/A3: Offset Printing Materials

13UY0132-3/A4: Knowing Offset Printing System and Web Offset Printing Machinery and Maintenance Applications

13UY0132-3/A5: Web Offset Printing Preparations and Printing Applications

ANNEX 2: Terms, Symbols and Abbreviations

Printing Control Strip: Control strips which ensure control of ground density, dot gain, trapping, printing setting, etc. properties of print by means of devices such as density-meter/spectrophotometer or visually.

High-grade paper: The paper with high cellulose content, whose surface is not coated,

Blanket: Rubber material with certain thicknesses which ensures transfer of ink onto paper,

CMYK: The color space consisting of cyan, magenta, yellow, and black (added later),

Doubling: Regional disproportion of image printed on the paper,

Density-meter: Electronic device, which is used to measure ink densities on printing control strips,

Plinth: Edge opposed to the one kept by shears in printed sheet,

Fixer: Chemical material which fixes the emulsion of mould onto mould,

Weighting pressure (offset): Pressure the mould, blanket, and press cylinders apply to each other,

Photo Cell: Electronic circuit element which senses whether paper enters into machine properly or not in printing machines,

Gray balance: Setting of gray color in print,

Reservoir water: Water used for transfer of ink onto printing material in balanced manner,

IPA: Isopropyl alcohol,

ISCO: International Standard Classification of Occupations,

Lightly glazed paper: Paper type whose wood pulp content and cellulose content are equal or in approximate values,

OHS: Occupational Health and Safety

Paper weight in gram: Weight of 1 m2 of paper in gram,

Mould: Material which contains the image to print and performs printing,

Personal Protective Equipment (PPE): All kinds of tools, instruments, appliances and devices which are worn, put on or hold by the worker and which protect the worker from one or more hazards arising from the work and effect the health and safety of the worker, and which are designed to suit such purpose,

Corrector: Chemical solution which ensures removing undesired parts on the mould whose copying process was completed,

Shear: Direction which the paper is kept by shearers in printing machine,

Roller: Rubber cylinder which conveys ink and / or water,

Ink: Homogenous dispersion of colorants within a binding system,

Humidifying Unit: Unit consists of water reservoir, trolley, and water-delivering rollers and which ensures smooth distribution of reservoir water in offset printing machines,

Offset printing (lithography): Printing technique which is based on the principle that water and oil push each other,

Blueprint: Trial print for control purpose,

Pantone: International Color Formula Guide which codes printing colors (pale and bright) and gives formulas of mixture (in percent and scale) (includes 1164 colors),

Pressurized strip: Paper-and plastic-based insulating materials which are put between printing machine blankets,

Punch: Punching process which ensures mould fit onto printing machine,

Register: The system which ensures printing moulds are fitted on each other on the paper in offset printing machines,

Guide (cross): Thin lines in “+” shape which allows visual control of fitting of print and colors on the paper,

Spectrophotometer:  Device measuring distribution of luminous energy reflected from color or printed ink image,

Trapping: The ratio of printed ink to accept other ink printed on it,

Web offset (rotary press): Offset printing system which is used to print mass circulation journals, books, newspapers, magazines, catalogues, yellow pages, brochures, etc. works in short times and large amounts as well as special coating, filming, cutting, folding, and adhering operations on the roll paper.

ANNEX 3: Ways of Horizontal and Vertical Progress At Occupation

Horizontal Progress

Candidate who is granted certificate for Web Offset Printing Operator (Level 3) National Qualification shall be exempted from “A1: Occupational Health and Safety, Environmental Safety, Quality”, “A2: Work Organization and Occupational Development”, and “A3: Offset Printing Materials” qualification items, which are common to offset printing operators group of level three when s/he applies for other offset printing operator qualification items with same level within validity period of her/his certificate.  The candidates who are successful in examination of other offset printing operator qualification items are entitled to get certificate for the occupation which they apply for and can progress in horizontal direction at that profession.

Vertical Progress

Vertical progress f Web Offset Printing Operator (Level 3) occupation is Web Offset Printing Operator (Level 4) occupation.

ANNEX 4: Assessor Criteria

Assessors to take role in assessment processes relating to Web Offset Printing Operator (Level 3) occupation should meet one of the following criteria:

- Having at least ten (5) years of experience as Web Offset Printing Operator (Level 4), having not suspended more than 5 years the occupation, and being at least graduate of high school,

- Trainers having experience in assessment applications and having worked at least 5 (five) years in printing sector or secondary or higher education institutions related to the occupation.

Assessors shall be provided with training on vocational qualification system, natural qualifications related to their task area, assessment, and quality assurance.

ANNEX 5(*): Institutions / organizations Contributing to Draft Qualification prior to Sending to Official Opinion

1. Ataşehir İbrahim Müteferrika Tarde Vocational High School (Kayhan Şen, Vice Principal)

2. Fatih İbrahim Müteferrika Trade Vocational High School (Turgut Yılmaz, Acting principal)

3. Marmara University (Assistant Prof. Hüseyin N. Beytut, Academic / Assistant Prof. Mehmet Oğuz, Academic / Doğan Tutak, Academic)

4. Milsan Printing and Publishing Co., Inc. (Osman YeşilProduction & Planning Superviser / Ersoy Türk, Sheet-fed Offset Chief/ Salim Ekim, Web Offset Chief

5. Ulubatlı Hasan Vocational High School (Abdülhamit Akçalar, Vice Principal)

ANNEX 6(*) Institutions / Organizations Which Draft Qualification was Sent for Opinion

- Ankara Chamber of Printing Houses Binders and Craftsmen

- A4 Offset Printing House

- ABA Printing Ind. Trade Co. Ltd.

- Acar Group Printing Ind. Trade Co. Inc.

- Ahmet Karademir İMAK Ofset Ltd.Şti

- Akyıldız Newspaper and Printing Co., Inc.

- Anatolian Association of Printing Publishing Packaging Development (ANAMAYA)

- Ankara Chamber of Industry

- Ankara Chamber of Commerce

- APA UNIPRINT Printing Industry & Trade Co., Inc.

- Arset Printing Ind. & Trade Co. Inc. A.Ş

- Ayşe Umur Umur Printing and Stationery Co. Inc.

- Printing Industry Training Foundation (BASEV)

- Association of Press Members (BASMEN)

- UNION of PRINTING AND PUBLISHING INDUSTRY

- Dizayn Printing Ind. Co. Ltd.

- Dr. Oktay Duran Vocational High School of printing

- Ebru Printing Publishing Ind. & Trade Co. Inc.

- Aegean Region Chamber of Industry (EBSO)

- Gazi University School of Technical Education Department of Printing Education

- Hak-İş Trade Union Confederation

- HALAMAN Printing

- İHLAS Printing Co., Inc.

- İkramat Offset Printing Graphic Trade Co. Ltd.

- Inter Printing Ind. Co. Ltd.

- Istanbul Aydın University Anatolian Vocational High School Printing and Publishing Technologies Program

- Istanbul Chamber of Tradesmen and Craftsmen

- Istanbul Fatih Ibrahim Müteferrika Technical Vocational High School

- Istanbul Chamber of Printing Houses

- Istanbul Offset Printing Publishing Ind. Trade Co. Inc.

- ISTANBUL CHAMBER of INDUSTRY

- ISTANBUL UNIVERSITY OF COMMERCE

- Istanbul University Vocational School of Higher Education Design Publishing and Printing Program

- Izmir Chamber of Printing Houses Tradesmen and Craftsmen

- Kelebek Printing Co., Ltd.

- Korozo Packaging Co., Inc.

- Small and Medium Industry Development and Supporting Administration (KOSGEB)

- Marmara University School of Technical Education Department of Printing Education

- Mas Printing Co., Inc.

- MATSİS – Printing Systems Co., Inc.

- MAVİ AĞAÇ CLTURE PUBLISHING Co., Ltd.

- MEGA Printing & Publishing Co., Inc.

- MGA Printing Graphic Agency Textile and Printing materials Ind. Trade Co., Ltd.

- Milsan Printing Industry Co., Inc.

- Association of Independent Industrialist and Businessmen

- Nesil Printing Co., Ltd.

- Orhan Çakıl Co., Ltd.

- PLATO Printing Advertising Informatics Consultancy Services Co.

- PRINT CENTER

- Association of Industrial Publishers

- SEN Etiket Matbaacılık Ltd. Şti.

- State Personnel Administration

- Ministry of Labor and Social Security, Directorate General of Occupational Health and Security

- Ministry of National Education, Life-Time Learning Directorate General

- Ministry of National Education, Occupational and Technical Education Directorate General

- Ministry of National Education, Head Council of Education and Morality

- Ministry of National Education, Innovation and Education Technologies Directorate General

- TİMAŞ Printing Publishing Ind. Co., Inc

- TTA A.Ş.

- Turkish Confederation of Revolutionary Trade Unions

- Confederation of Turkish Tradesmen and Craftsmen - TESK

- Turkish Exporters Assembly

- Turkish Statistics Institution

- Turkish Labor Institution

- Turkish Confederation of Trade Unions

- Turkish Confederation of Employer Associations

- Turkish Union of Chambers and Exchange Commodities -TOBB

- VERİTAS Printing Center

- Yeni Asya Journalism and Printing Co., Inc.

- Board of Higher Education (YOK)

ANNEX 7(*): Opinions of Institutions / Organizations on Draft Qualification and Form Relating to Evaluation of These Opinions

.

| National Qualification Level: |  |  | Web Offset Printing Operator (Level 3) |  |  |
| --- | --- | --- | --- | --- | --- |
| Deadline for Submission of Opinion: |  |  | April 22nd, 2010 |  |  |
| Name / Trade Name of Institution / Person Giving Opinion: |  |  | - Rize Chamber of Commerce and Industry Chamber - Turkish Statistical Institute (TUIK) |  |  |
| E-Mail |  |  | - |  |  |
| Phone: |  |  | - |  |  |
| Fax: |  |  | - |  |  |
| This form is used to receive and evaluate the opinions of related institutions in order to improve transparency and participation during process of preparation of National Qualification and establish an objective and nationally recognized Qualification. Opinions of real or legal persons, who are thought to contribute to process, can be obtained by copying and sending to them. Please send the form by mail to the address of Istanbul Ticaret Odası İTO TEST Merkezi Reşadiye Caddesi 34112 Eminönü İSTANBUL or by e-mail to recep.dayioz@ito.org.tr. Thanks for your opinions and contributions. |  |  |  |  |  |
| No | Part, Row, Page in the Standard | Opinions |  | To be filled by Institution / Organization preparing the standard / qualification |  |
|  |  |  |  | Evaluation | Correction |
| Rize Chamber of Commerce and Industry / Gafur KARALI / General Secretary |  |  |  |  |  |
| 1 | ALL | Suitable |  | - | - |
| Turkish Statistics Institution (TUIK) / Döndü ARICAN / dondu.arican@tuik.gov.tr |  |  |  |  |  |
| 2 | ALL | No opinion |  |  |  |

ANNEX 8(*):  Information Regarding Pilot Study Carried Out for Determining Criteria to Apply in Qualification

The exam was conducted in printing house determined by ITO-TEST. 5 candidates took Web Offset Printing Operator (Level 3) Theoretical Assessment Exam and 2 were successful in all items while 3 candidates failed at least one item. Item which candidates failed is dominantly bag tasks. Candidates were given 2 minutes per question and this time is enough to answer questions. However, it was observed that candidates are not very sufficient in theoretical questions and they had difficulty in understanding and answering such questions.

{0>Bu nedenle adayların başarı oranını yükseltme için birim soru sayısının 8-10 sorudan az olmamasının uygun olacağı söylenebilir.<}0{>Therefore, it can be said that the number of questions asked in this item should not be less than 8-10 in order to increase success of candidates.

{0>Performans değerlendirmede öngörülen değerlendirme ölçütlerinin çokluğu ve uygulanması gereken işlemlerin uzunluğu sınav süresinin her aday için yaklaşık 3-3,5 saat gibi bir sürede tamamlanmasına neden olmuştur.<}0{>Since there are many assessment criteria and procedures to apply take very long time the examination take long time for each of candidates; namely, not less than 3-3.5 hours.<0} {0>Bu sürede gerçekleştirilmesi beklenen işlemlerin özellikle Seviye 3 düzeyinde makinenin durdurulup müdahale gerektiren işlemler olmaması sonucuna ulaşılmış olup yeterlilik dokümanındaki her bir birimin EK [A5]-2(*):Yeterlilik Biriminde Belirtilen Değerlendirme Araçları İle Ölçülen Başarım Ölçütlerine İlişkin Tablo’sunda gerekli düzenlemeler yapılmıştır.<}0{>

It was concluded that processes expected especially from Level 3 should not be processes which requires stopping and intervening with the machine during examination and required arrangements were made in ANNEX [A5]-2: Table Regarding Performance Criteria Assessed by Assessment Tools Specified in Qualification Item of each item in qualification document relating to operations expected within this time.

2 candidates took performance based assessment and both were successful. It was observed that candidates took higher scores in performance based assessment although their theoretical scores are low.

| [WEB OFFSET PRINTING OPERATOR] LEVEL [3] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Attendant | Qualification Item | Theoretical Test |  |  |  | Performance Based Exam |  |
|  |  | (Min. 60 point) |  |  |  | (Min. 70 point) |  |
|  |  | Number of Questions | Number of Correct Answers | Score (%) | Result (P/F) | Score | Result (P/F) |
| Candidate 1 | A 1 | 5 | 1 | 20 | F | 75 | F |
|  | A2 | 5 | 3 | 60 | P | - |  |
|  | A3 | 5 | 3 | 60 | P | 100 | P |
|  | A4 | 5 | 4 | 80 | P | 92,3 | P |
|  | A5 | 5 | 4 | 80 | P | 85 | P |
| Candidate 2 | A 1 | 5 | 3 | 60 | P | 100 | P |
|  | A2 | 5 | 4 | 80 | P | - |  |
|  | A3 | 5 | 3 | 60 | P | 100 | P |
|  | A4 | 5 | 3 | 60 | P | 85,2 | P |
|  | A5 | 5 | 3 | 60 | P | 90 | P |
| Candidate 3 | A 1 | 5 | 3 | 40 | C |  |  |
|  | A2 | 5 | 3 | 60 | P |  |  |
|  | A3 | 5 | 3 | 60 | P |  |  |
|  | A4 | 5 | 3 | 60 | P |  |  |
|  | A5 | 5 | 3 | 60 | P |  |  |
| Candidate 4 | A 1 | 5 | 4 | 80 | P |  |  |
|  | A2 | 5 | 3 | 60 | P |  |  |
|  | A3 | 5 | 4 | 80 | P |  |  |
|  | A4 | 5 | 4 | 80 | P |  |  |
|  | A5 | 5 | 3 | 60 | P |  |  |
| Candidate 5 | A 1 | 5 | 3 | 60 | P |  |  |
|  | A2 | 5 | 4 | 80 | P |  |  |
|  | A3 | 5 | 2 | 40 | C |  |  |
|  | A4 | 5 | 3 | 60 | P |  |  |
|  | A5 | 5 | 2 | 60 | P |  |  |

ANNEX 9(*): Explanations Regarding Conditions to Take Qualification Exam and Certificate Validity Period